# TABLE OF CONTENTS

# SECTION DESCRIPTION                                                                    PAGE NO

TABLE OF CONTENTS  ……………………………………..…………….	         i

REVISION PAGE  …………………………………………………………..                  ii

1.	GENERAL / PURPOSE  …………………………………………………….               1-1

2.	PROJECT SETUP  …………………………………………………………..               2-1

3.	FILE MANAGEMENT AND TRACKING  ………………………………..               3-1

4.	GRAPHICS STANDARDS  ………………………………………………...               4-1

5.	PLOTTING / PRINTING  …………………………………………………..               5-1

6.	DRAFTING PROCEDURES  ………………………………………………               6-1

REVISION PAGE

This page will be used to track all revisions made to this document.  This page will be distributed along with any replacement pages.

Initial Release – version 1.0  …………………………………..…  June 26, 1998

Version 2.0  …………………………………………………...   August 11, 1998

New title, incorporation of City and Sverdrup CADD standards, and revision of version 1.0.

Version 2.1 …………………………………………………...   August 27, 1998

Eliminated cmntsht1.dgn reference file, made 3-Engineering the default font instead of 1-Working, added instructions to section 6.

Version 2.2 …………………………………………………...   October 2, 1998

A few minor changes, clarified and added some instructions to section 6.

## 1.  GENERAL / PURPOSE

1.1 PURPOSE

This manual was developed to establish the CADD standards that will be used for COMNET / Loop Diagrams.  These standards meet the CADD guidelines provided by the City of San Diego.  For the items in which the City did not provide specifications, the Sverdrup Civil CADD Standards manual was followed.  All other specifications were made to best suit the purposes of COMNET.  Also included are procedures for developing the various sheet types and drafting tips to help those unfamiliar with Bentley MicroStation.

2.  PROJECT SETUP

2.1.  DIRECTORY STRUCTURE / FILE NAMING

2.1.1. Directory Structure

Directory						Description

J:\Comnet\Loops\

proj_std\				Project standards files

cell\			Cell library

config\			Project configuration file .PCF

level\			Level structure file .LVL

macro\			Macros (used for plotting)

manual##\		Latest version ## of CADD manual

plot\			Plot configuration file .INI

Plotter driver file .PLT

Pen table .TBL

refer\			Reference files

seed\			Seed files

template\		Templates

trnslate\		For translation of .DWG files

project\				Project design files

project name\		CBF, CSRS, CWT, PGDUA, PS64, etc.

as_built\	Final as-built loops (read only)

dl\	Data link loops

hw\	Hard wired loops

draft\		Draft version of loops (read and write)

dl\	Data link loops

hw\	Hard wired loops

2.1.2.   File Naming Convention

Every loop sheet file will have a distinct 8-character file name with a 3-character extension.  This will prevent filename conflicts in case loops from different projects are combined into the same directory.  The 8-character filename allows the files to be compatible with older computer formats.  All filenames should use lower case for compatibility with Unix systems.

2.1.2.1.  Character Position Significance W/extension

Loop sheet files – Example of a loop sheet file name: p10y2019.001

1		Project letter, example: p – Point Loma, m - MBC

2-3		Area No.

4		Loop type

5-8		Four digit loop no. (use zeros in front if needed)

.###		3-digit no. for sheet no., example 001, 002, 003, etc.

Loop reference files – Example of loop reference file name: p10y2019.ref

1-8		Same as above

.ref		Reference file extension

2.1.2.2.  Seed Files

cmntseed.dgn		Standard seed file (“blank”) for COMNET loops

2.1.2.3.  Border Files

cmntbord.dgn		Standard border file for COMNET loops

2.2 SEED FILES

A seed file is a “blank” design file that has all the parameters such as working units, global origin, coordinate readout, locks, text/font settings, view attributes, level names and symbology, and grid settings.  The seed file cmntseed.dgn will be used to begin all COMNET loop sheet and loop reference files.

    - Global Origin

The global origin defines the point on the design plane that will define the X and Y axes.  The GO will be set to the exact center of the design plane.

Global origin = 178956.970667, 178956.970667

Note:  This set by the KEY-IN input “GO = -178956.970667, -178956.970667”.  The GO can also be check by entering “GO = ?”.

The global origin at the center the design plane meets both City of San Diego, and Sverdrup CADD standards.

2.2.2.  Working Units

The units MicroStation uses to describe distance in a coordinate system are known as working units.  The units consist of  master units (MU), sub units (SU), and positional units (PU).  Non-scaled schematic drawings use the following:

Schematic/non-scaled (English)

MU		1 ft.

SU		12 in.

PU		1000

2.2.3.  Coordinate Readout

Coordinates

Format		Sub Units

Accuracy	0.123

Angles

Mode		Conventional

Format		DD.DDDD

Accuracy	0.1234

2.2.4.  Locks

Grid Lock	ON		Text Node Lock	OFF

Level Lock	OFF		Graphic Group	OFF

Boresite	ON		ACS Plane		OFF

Fence Mode	Inside

Snap Lock	ON

Mode		Keypoint

Divisor	2

Association	OFF

ACS Plane	OFF		Depth Lock		OFF

Axis Lock	OFF

Start Angle	0.00

Increment	30.00

Unit Lock	OFF

Distance	1.00

Isometric Lock	OFF

Isometric Plane	Top

2.2.5.  Text/Font

Standard Font: 3 - Engineering

This font should be used on all COMNET loop sheets.  It is a straight (not slanted), proportionally spaced, non-area fill (stick) font.  All text should be written using all CAPS.  The text sizes should be multiples of 0.03 inches.

Text Sizes (feet: inches):			Use

0:0.06				Minor border info headings.  Smallest used.

0:0.09				Standard size used for most information.

0:0.27				For “LOOP NO:” at top of sheet 1.

0:0.36				For actual loop no. at top of sheet 1.

The line spacing should always be set at one-half the text size.  Example:  The line spacing for 0:0.09 should be set at 0:0.045.  This is necessary for creating a text node (more than one line).

2.2.6.  View Attributes

Applies to all views:

ACS Traid	OFF		Fast Ref Clipping	OFF

Background	OFF		Fill			ON

Camera	OFF		Grid			ON

Constructions	OFF		Level Symbology	ON

Dimensions	ON		Line Styles		ON

Dynamics	ON		Line Weights		ON

Data Fields	OFF		Patterns		ON

Fast Cells	OFF		Ref Boundaries	OFF

Fast Curves	OFF		Text			ON

Fast Fonts	OFF		Text Nodes		OFF

2.3.  LEVEL STRUCTURE

Link from j:\comnet\loops\proj_std\manual22\cmntref.xls

Link from j:\comnet\loops\proj_std\manual20\cmntbord.xls

Link from j:\comnet\loops\proj_std\manual20\cmntdcs.xls

2.4. WORKSPACE

A workspace is a custom MicroStation environment or configuration.  The workspace consists of three components – project, user interface, and preferences.  The project configuration file (.pcf) is the only setting that has a COMNET standard.  When first opening MicroStation, the workspace settings box should appear as shown:

The project setting must say COMNET or the configuration settings will not take effect.  If you do not see COMNET listed under Projects, you need to use Microsoft Explorer to copy j:\comnet\loops\proj_std\config\comnet.pcf to your personal hard drive at c:\Win32app\ustation\Config\Project\comnet.pcf.

3.  FILE MANAGEMENT AND TRACKING

3.1. FILE STORAGE

All of our loop reference and loop sheet files are to be stored on the  j:\Comnet\Loops\project\ directory under the correct project folder name.  The files are kept in this central location so that they can be backed up nightly by the network.  Storing design files on your PC’s c:\ drive is a dangerous practice as you risk losing hundreds of hours of work with one crash of your hard drive.

  - FILE MODIFICATION

Loop reference and loop sheet files under the j:\Comnet\Loops\project directory may be modified only by those who have been assign to work on that particular project.  Those who wish to view a file that is not in their project should check the “Open Design Files Read-Only” option on the MicroStation File Manager.  This will prevent accidental modification of information.

The design reference files and CADD configuration files found under the j:\Comnet\Loops\proj_std\ directory should only be modified by the CADD manager.  This will promote uniformity in the content and appearance of our drawings.

  - FILE LOGS

It is the responsibility of the each CADD user to keep an updated Excel spreadsheet that logs each loop reference and loop sheet file for a particular project.  The information of this file should include the filename, file description, date created, point names contained, and any other pertinent information.  This file will be printed on 11 X 17 paper and used as a coversheet for files that are sent to the field.  A blank template Excel spreadsheet for this purpose can be found in j:\Comnet\Loops\proj_std\manual22\blnk_log.xls.

  - FILE BACKUP

In addition to the nightly backup done automatically by the network, the CADD manager shall periodically copy all the directories and files under j:\Comnet\Loops\ using a Zip drive.

4. GRAPHICS STANDARDS

  - TEXT

All text shall follow the standards given in section 2.2.5.  In most cases, the justification should be set to CENTER-CENTER.  In some cases, LEFT-CENTER should used when text needs to be aligned on the left side.

  - LINE WEIGHTS

The line weight scheme, which is automatically implemented in the level symbology, is designed to make drawing specific information stand out compared to less important information.  This makes the drawings more attractive and easier to read.  The more important information is assigned to weight-1 while less important is assigned weight-0.

  - CELL LIBRARIES

COMNET will use one cell library for all projects.  The library, j:\Comnet\Loops\proj_std\cell\cmntcell.cel, is already attached to the COMNET seed file.  The purpose of the library is to save time and promote uniformity in the appearance of the drawings.  The library should be checked each time that an element for a drawing is needed and should be added to when a new element is created.  Logical names and descriptions should be used for each cell to make it easier for others to find and use the cell.

5.  PLOTTING / PRINTING

  - PLOT SETTINGS

Each CADD station should have its document default settings for the printer, ske_hp5si on sd set to:

Paper size:	Tabloid

Paper souce:	Automatically Select

Orientation:	Landscape

These settings are made using the Control Panel in Windows ’95.  Setting the document defaults here will also cause MicroStation default to these settings also.

  - MACROS

All CADD drawings will be printed using the cmntplot macro in MicroStation.  This macro automatically fences and plots the drawing using the plot configuration file, pen table, and plot driver that has been previously configured to meet the purposes of COMNET.

  - PEN TABLE

A pen table is designed to customize the appearance of plots.  The pen table, j:\Comnet\Loops\proj_std\plot\cmntpen.tbl, was configured to make those elements drawn as weight-1 appear darker that those drawn as weight-0.

6.  DRAFTING PROCEDURES

  - REFERENCE FILES

Reference files are used when the same information needs to be used repeatedly.  A reference file is used by attaching it to the file that you are currently working on.  The levels of the reference file can then be turned on or off to reveal the information that is needed for your drawing.

6.1.1.  Border Reference File

The border reference file, j:\Comnet\Loops\proj_std\refer\cmntbord.dgn, outlines the border and contains additional information for the SKE logo, section headings, sheet #, of #, project name, and project number.  See section 2.3. to see the level assignments for this file.

6.1.2.  DCS Reference File

The DCS reference file, j:\Comnet\Loops\proj_std\refer\cmntdcs.dgn, contains the WPCD PCM card and DCS software information that appears on sheets 2 and 3. See section 2.3. to see the level assignments for this file.

6.1.3.  Loop Reference File

Each loop will have it’s own loop reference file.  This file contains all the information that is specific to that loop.  The COMNET seed file, j:\Comnet\Loops\proj_std\seed\cmntref.dgn, will be used to produce a .ref file.

6.2.  SHEET FILES

Sheet files are used strictly for generating plots and carry the extension .001, .002, .003, etc.  They are produced to display a single sheet by attaching the loop reference file, .ref, along with the reference files, cmntbord.dgn and cmntdcs.dgn.

6.3.  DRAFTING TIPS

### Setting Up Workspace

###### Toolboxes

How you set up your toolboxes is a matter of personal preference, but following these guidelines will give you the basic tools you’ll need to get started.

The Standard toolbox

The Primary toolbox

Select each of the above toolboxes from the Tools menu.  If they appear as they do above and are floating on top of your design, then simply click and drag them to the top of the screen to dock them.  That way they are out of the way and easily accessible.

The Main toolbox

Select Tool Boxes from the Tools menu.  A dialog box will appear.  Scroll down the list to find the Main toolbox.  Check it, then click OK.  The Main toolbox will appear as it looks on the left.  Click and drag the box to the left-hand side of the screen to dock it.

Viewing Designs

Windows

When you first open a design file, you may notice that there are two viewing windows to choose from.  It is not necessary to use more than one window to view a design, but if you would like to work on different parts of your drawing at the same time, then you should use multiple windows.

Select Open/Close from the Window menu to get this dialog box.

Levels

Levels are a way to distinguish between different elements of your loop diagram.  For instance, your could draw the framework on level 1, the circuit elements on level 2, and the text on level 3.  The advantage of using levels is the ability to isolate certain elements for viewing and manipulation.

Select Levels / Display from the settings menu.  Highlight the levels that you want displayed and click Apply.

2D view control

Navigating among different sections of your drawing can be done quickly by using the 2D view control.  This toolbar can be called up and docked just like the Main toolbar shown previously, but it is already included at the bottom of every viewing window.

Undocked version of the 2D view control.

Most useful features of this toolbar:

Update View: This feature refreshes your display window.

Zoom In/Out

Fit View: Use when you need to see the entire drawing at one time.

Window Area: Similar to zoom, but easier to use.  Use with Fit View to get anywhere

View Previous/Next: Good to use after zooming in on an item.

Drafting Designs

Accudraw

Most of the time, it is necessary to be able to draw straight lines or place elements at right angles to each other. Accudraw is the tool that makes it easy.

This tool is found in the Primary toolbox at the top of the screen.

The next description will show an example of how to use Accudraw.

Using cell library

Most of the devices that you will need to represent in your drawings have already been drawn and placed in the cell library.  To open the library, select Cells from the Select menu.

Once the library is opened, then choose the desired cell by clicking on its name.  Then click the Placement button to tell the library that you want that cell available for placement in a drawing.  The graphic from the previous page shows the selection of a relay.

The Place Active Cell icon is found in the Main toolbox to the left of the screen.

Click the icon and you will see your selected device appear.  Move the device to the desired location with your mouse and click to place it.

You can also access cells by using the Cell Selector tool found under the Utilities menu.

By using Accudraw, one element can be placed exactly below another.

Placing text

The first thing to consider when placing text is what size to use.  Almost all of the text used in our drawings uses an ideal size of 0.09 x 0.09.  But that information is not needed if there is text of the correct size already existing on the drawing you are working on.

Click the Text Attributes icon located on the Main toolbar on the left.  Then double click on any text that has the desired size.  Now using the Place Text tool will produce the correct size text

Manipulating Designs

Selecting Elements

The best way to speed up your drafting time is to learn how to move, copy, delete, or change many elements at the same time.

The selection arrow can be found in the Main toolbox on the left-hand side of the screen.

To select a single element, simply click on it.  You know an item has been selected if small white boxes appear at its corners.

Multiple items can be selected in two ways.  First, if you would like to select a group of items that are near each other, you can click and drag the arrow to form a box around them.  All elements that are completely surrounded by the box are selected.  Second, if the box method is not convenient, you can hold down the CTRL key as you click on each element you would like to select.

A selected element can have any number of things done to it such as moving, copying, changing color/weight, or deleting.

Changing text

When making repetitive changes such as changing the loop number or the device schedule on Page 1, the Find/Replace Text command on the Edit menu is useful.  It is important, however, not to use the Change All button unless you are absolutely sure that it is not going to make unwanted changes.  It usually better to alternate between the Find and Change buttons in order to view each change as it happens.

The Change Text icon, found in the Main toolbox can be used to change a single text item.

6.4.  DRAFTING PROCEDURES

This section outlines the procedure for producing loop reference files, *.ref, along with their corresponding loop sheet files, *.001, *002, and *003.  The following illustration shows the relationship between these files:

Step by step for creating loops:

- Make sure your workspace is set-up as specified in Section 2.4.

- Print out your own copies of cmntref.xls, cmntbord.xls, and cmntdcs.xls found in the j:\Comnet\Loops\proj_std\manual21\ directory for a reference while drafting.

- Create a *.ref file:

Select New from the File menu.  Make sure the seed file at the bottom of the window reads j:\comnet\loops\proj_std\seed\cmntref.dgn.  If it does not, hit the SELECT button to the right.  A new window should appear and then you can select MS_DESIGNSEED from the Directory Menu and then select the correct file and click OK.  Use the folder selector window to find proper directory in the j:\Comnet\Loops\project\ directory that the new file will reside.  In the field for filenames, type in an 8 character filename following section 2.1.2. with a *.ref extension.  Click OK and the file is then created using the chosen seed file.  Click OK again to open the file.

- Set reference file levels:

The reference files cmntbord.dgn and cmntdcs.dgn should already be attached to the new file.  Select Reference from the File menu.  Click on cmntbord.dgn, then select Levels from the Settings menu.  Refer to your copy of the Excel spreadsheet cmntbord.xls for the next step.  Check views 1,2,and 3 corresponding to sheets 1,2, and 3 of your drawing and make sure that the default levels settings (the levels that are already turned on) apply to your drawing.  If your need to changes to these settings, just turn on or off the appropriate levels and the Click Apply, not All, to make the changes for each view.  Follow the previous steps for cmntdcs.dgn also.  If you are not going use this reference file in your drawing, then click off the Display box.

5.  Drafting:

To draw an element, place text, or place a cell, make sure that your active level is set to the appropriate level.  For instance, if you want to fill in the grid information for sheet 1, the active level must be set to 10.  There are three view windows to work with, each corresponding to the three sheets of your drawing.  When you are done working on one sheet, close the drawing and then open it again.  This will restore the proper level settings for each sheet.  Then you can move on to the next sheet.

- Create *.001 sheet file for plotting:

Click New from the file menu.  Make sure the seed file is set to the *.ref file that was created in step 4.  In the field, type in the correct filename with a *.001 extension. Click OK to create the file, then Click OK to open the file.  Choose Select All from the edit menu, then hit the Delete key to remove all the information contained in the file.  Then select Reference from the File menu.  Then select Attach from the Tools menu and then find and select the *.ref file created in step 4.  Enter a logical name for the reference file and then click OK.  Uncheck the boxes for SNAP and LOCATE since they aren’t needed.  You can now close the reference window.  Now use the Windows menu to close windows 2 and 3.  Then select Save Settings from the File menu.  Now the sheet file is available for plotting.

6.  Create *.002 and *.003 for plotting:

Click New from the File menu.  Make sure the seed file is set to the *.001 that was created in step 5.  In the field, type in the correct filename with a *.002 or *.003 extension.  Click OK to create the file, then Click OK again to open the file.  Use the Windows menu to make sure that only Window 2 or Window 3 is open.  Then select Save Settings from the File menu.  Now the sheet file is available for plotting.